CNC Heavy duty Roll Turning lathe suitable for roughing and finishing section mill rolls with profiles of the cast iron roll or high speed steel or tungsten carbide roll etc.

The heavy duty roll turning lathe are suitable for workpiece up to the following data:

Max. Diameter to be machining : 550mm

Max. center distance:2500mm

Max roll weigh on center:3500Kg

CNC controller: siemens 828D

Optional function 1

Special software for programming shop turn machining step programming.

Optional function 2

The turret block can be selected with the NC quartet turret. It can fix 4 shanks at the same time. it’s very good for machining rough and finish with different shanks.

Description:

The workpiece are accommodated between the 4-jar faceplate of the headstock and the center of the tailstock.

The CK8450*2500 CNC heavy duty turning lathe are the series of  5-bedways turning lathes has been especially designed for rolling cylinder maintenance. The CK8450*2500 CNC heavy duty turning lathes are strong and reliable, with superior rigidity given by the oversized components and 5 guideways design. There are 2 guideways for the tailstock, 2 guideways for the saddle in the level surface, at the same time there have 1 guideway on the vertical surface link to the end of the saddle beside the operation side.

MAIN TECHNICAL CHARACTERISTIC of The CK8470*2500 CNC heavy duty roll turning lathe

| CAPACITY |  |
| --- | --- |
| Height of center over bed guideways | 425 mm |
| Distance between center | 2500 mm |
| Max. diameter of the roll | 550 mm |
| Bed width | 920 mm |
| Number of guideways | 4 + 1 |
| Max workpiece weight supported between center without steadies | 3500 kg |

Headstock

Spindle, shafts and transmission gears made in 18 Ni Cr Mo5, completely hardened and ground. Spindle mounted on journals which are provided with high precision SKF bearings, specially selected for spindles. 4 Jaws independent chuck, cast steel  body, diam. 650mm. A.C. brushless spindle motor controlled with VFD. Stepless variation of the spindle speed within each speed range. Lubrication system with continuous forced lubrication of all components of the headstock, complete with safety device which intervenes in case of defective operation.

| HEADSTOCK |  |
| --- | --- |
| Spindle nose | Metric #80 |
| Inner diameter of front bearing | 240 mm |
| Spindle motor A. C. AC SERVOR | 30kW |
| Number of spindle speed ranges | 2 |
| Variation of r.p.m. at different ranges | r.p.m.5~350 |
| 4 jaws independent chuck cast steel body, 550mm, |  |

Tailstock

Tailstock for these lathes is specially designed to reduce the distance between centre and the large bearings in order to increase stability and avoid vibrations.

| TAILSTOCK |  |
| --- | --- |
| Quill diameter | 220 mm |
| Quill travel | 250 mm |
| Quill travel control | Motorized |
| Tailstock travel control | Motorized |