PET ONE WAY VISION FILM SF1552 High Quality Vinyl For Digital Printing TECHNICAL DATA SHEET

PRODUCT DESCRIPTION:

Film:200 microns white/black PET film

Color :White

Surface: Matt

PRINTING METHOD:

Compatible with Solvent Ink, Eco-solvent, Latex

TECHNICAL DATA:

| CHARACTERISTIC | TEST METHOD | AVERAGE VALUES |
| --- | --- | --- |
| Film Weight | ISO 4591:1992 | 200micron white/black PET film |
| Release paper |  | 50 micron white PET |
| Dimensional stability | FINAT TM14 | no shrinkage in cross direction, in length |
| Temperature resistance |  | Adhered to aluminum. -20℃ to +100℃, no variation |
| Diameter of hole |  | 1.50mm |
| Perforating ratio |  | 20% |
| Shelf life |  | 1 year |
| Storage |  | One year out of direct sunlight at 23℃ and 50% humidity |
| Application temperature |  | Min.+10℃ |
| Weathering |  | 3 year |

Attention: After printing the ink must be thoroughly dry in order to avoid any affect on the later combination with the laminate. Surfaces to which the material will be applied must be thoroughly cleaned from dust, grease or any contamination which could affect the adhesion of the material.

The statements in this information sheet are based upon our knowledge and practical experience. This data is intended only as a source of information and is given without guarantee and does not constitute a warranty. Due to the wide variety of possible uses and applications customers should independently determine the suitability of this material for their specific purpose, prior to use.