MOQ: 2000

Lite Series  1.54”

Model: AES-0154/L  4 color is available

Minimal display: suitable for accessories, pharmaceuticals, perfumery, hygiene and beauty

Specifications

| Product Dimensions (mm*mm*mm) | 53.5*38.8*15 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 26.9x26.9 |  |
| Weight (g) | 22 |  |
| Case Color | white |  |
| Display Size (inch) | 1.54 |  |
| Resolution (pixel) | 200*200 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*375mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 18,000 labels per hour |

MOQ: 5000

Lite Series  2.13”

Model: AES-0213/L   4 color is available

Label with simple design. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 72.8*34.5*13 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 48x23.1 |  |
| Weight (g) | 33 |  |
| Case Color | white |  |
| Display Size (inch) | 2.13 |  |
| Resolution (pixel) | 250*122 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 18,000 labels per hour |

MOQ: 5000

Lite Series  2.66”

Model: AES-0266/L   4 color is available

Label with simple design. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 90.8×42.9*13 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 59.5x30 |  |
| Weight (g) | 36 |  |
| Case Color | white |  |
| Display Size (inch) | 2.66 |  |
| Resolution (pixel) | 296*152 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 15,000 labels per hour |

MOQ: 5000

Lite Series  2.9”

Model: AES-0290/L    4 color is available

Label with simple design. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 90.8×42.9*13 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 66x28.5 |  |
| Weight (g) | 39 |  |
| Case Color | white |  |
| Display Size (inch) | 2.9 |  |
| Resolution (pixel) | 296*128 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 15,000 labels per hour |

MOQ: 5000

Lite Series  4.2”

Model: AES-0420/L

Perfect for promotion stand, fruit & vegetables and appliances section.

Specifications

| Product Dimensions (mm*mm*mm) | 106*86*14.1 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 84x63 |  |
| Weight (g) | 83 |  |
| Case Color | white |  |
| Display Size (inch) | 4.2 |  |
| Resolution (pixel) | 400*300 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 3*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 9,000 labels per hour |

MOQ: 2000

Lite Series  7.5”

Model: AES-0750/L

Perfect for promotion stand, fruit & vegetables and appliances section.

Specifications

| Product Dimensions (mm*mm*mm) | 185*121*14 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 162.6x97.3 |  |
| Weight (g) | 201 |  |
| Case Color | white |  |
| Display Size (inch) | 7.5 |  |
| Resolution (pixel) | 800*480 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 4*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 6,000 labels per hour |

MOQ: 5000

Slim Series  2.13”

Model: AES-0213/S   4 color is available

Slim design with protective cover. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 71*34.5*11.2 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 48x23.1 |  |
| Weight (g) | 32 |  |
| Case Color | White, black |  |
| Display Size (inch) | 2.13 |  |
| Resolution (pixel) | 250*122 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 18,000 labels per hour |

MOQ: 5000

Slim Series  2.66”

Model: AES-0266/S    4 color is available

Slim design with protective cover. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 90.7×42.8*11.2 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 59.5x30 |  |
| Weight (g) | 38 |  |
| Case Color | White |  |
| Display Size (inch) | 2.66 |  |
| Resolution (pixel) | 296*152 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 15,000 labels per hour |

MOQ: 5000

Slim Series  2.9”

Model: AES-0290/S   4 color is available

Slim design with protective cover. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 90*42*11.2 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 66x28.5 |  |
| Weight (g) | 39 |  |
| Case Color | White, black |  |
| Display Size (inch) | 2.9 |  |
| Resolution (pixel) | 296*128 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 15,000 labels per hour |

MOQ: 5000

Slim Series  4.2”

Model: AES-0420/S

Perfect for promotion stand, fruit & vegetables and appliances section.

Specifications

| Product Dimensions (mm*mm*mm) | 118*83.8*11.2 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 84x63 |  |
| Weight (g) | 83 |  |
| Case Color | White, black |  |
| Display Size (inch) | 4.2 |  |
| Resolution (pixel) | 400*300 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 3*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 9,000 labels per hour |

MOQ: 5000

Slim Series  7.5”

Model: AES-0750/S

Perfect for promotion stand, fruit & vegetables and appliances section.

Specifications

| Product Dimensions (mm*mm*mm) | 183*118*11.2 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 162.6x97.3 |  |
| Weight (g) | 201 |  |
| Case Color | white |  |
| Display Size (inch) | 7.5 |  |
| Resolution (pixel) | 800*480 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 4*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 6,000 labels per hour |

Freezer Series  2.66”

Model: AES-0266/F

Perfect to be used in freezers.

Specifications

| Product Dimensions (mm*mm*mm) | 85.9*41.9*9.1 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 59.5x30 |  |
| Weight (g) | 32 |  |
| Case Color | White |  |
| Display Size (inch) | 2.66 |  |
| Resolution (pixel) | 296*152 |  |
| DPI | 183 |  |
| Display color | Black and white |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 1200mAh Polymer battery |  |
| Changeable Battery | Not Changeable |  |
| Working Temperature (˚C) | -20 C°to + 40 C° |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP67 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 15,000 labels per hour |

MOQ: 2000

Rock Series  2.13”

Model: AES-0213/R

Slim design with protective cover. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 71*34.5*11.2 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 48x23.1 |  |
| Weight (g) | 32 |  |
| Case Color | White, black |  |
| Display Size (inch) | 2.13 |  |
| Resolution (pixel) | 250*122 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Universal single button battery (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 18,000 labels per hour |

MOQ: 2000

Rock Series  2.9”

Model: AES-0290/R

Slim design with protective cover. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 90*42*11.2 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 66x28.5 |  |
| Weight (g) | 39 |  |
| Case Color | White, black |  |
| Display Size (inch) | 2.9 |  |
| Resolution (pixel) | 296*128 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Universal single button battery (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 15,000 labels per hour |

MOQ: 2000

Rock Series  3.5”

Model: AES-0350/R   4 color is available

Slim design with protective cover. Perfect in every type of department.

Specifications

| Product Dimensions (mm*mm*mm) | 103.5×51×12.9 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 79x38 |  |
| Weight (g) | 68 |  |
| Case Color | White, black |  |
| Display Size (inch) | 3.5 |  |
| Resolution (pixel) | 384*184 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 2*600mAh CR2450 |  |
| Changeable Battery | Universal single button battery (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 12,000 labels per hour |

MOQ: 2000

Rock Series  4.2”

Model: AES-0420/R

Perfect for promotion stand, fruit & vegetables and appliances section.

Specifications

| Product Dimensions (mm*mm*mm) | 101*88.5*12.9 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 84x63 |  |
| Weight (g) | 83 |  |
| Case Color | White, black |  |
| Display Size (inch) | 4.2 |  |
| Resolution (pixel) | 400*300 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 4*600mAh CR2450 |  |
| Changeable Battery | Universal single button battery (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 9,000 labels per hour |

MOQ: 2000

Rock Series  5.83”

Model: AES-0583/R

Perfect for promotion stand, fruit & vegetables and appliances section.

Specifications

| Product Dimensions (mm*mm*mm) | 136*112*12.9 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 119x88 |  |
| Weight (g) | 105 |  |
| Case Color | White, black |  |
| Display Size (inch) | 5.83 |  |
| Resolution (pixel) | 648*480 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 4*600mAh CR2450 |  |
| Changeable Battery | Universal single button battery (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 9,000 labels per hour |

MOQ: 2000

Rock Series  10.2”

Model: AES-0102/R

Perfect for promotion stand, fruit & vegetables and appliances section.

Specifications

| Product Dimensions (mm*mm*mm) | 234.6×170.7×12.9 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 215.52*143.86 |  |
| Weight (g) | 220 |  |
| Case Color | White, black |  |
| Display Size (inch) | 10.2 |  |
| Resolution (pixel) | 960*640 |  |
| DPI | 183 |  |
| Display color | BW, BWR, BWRY |  |
| NFC | None |  |
| LED Flash | None |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 8*600mAh CR2450 |  |
| Changeable Battery | Universal single button battery (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 6,000 labels per hour |

7 color ESL  7.3”

Model: AES-0730/SC

Perfect for promotion stand, fruit & vegetables and appliances section.

Specifications

| Product Dimensions (mm*mm*mm) | 183*118*11.2 |  |
| --- | --- | --- |
| Active Display Area (mm*mm) | 160x96 |  |
| Weight (g) | 201 |  |
| Case Color | white |  |
| Display Size (inch) | 7.3 |  |
| Resolution (pixel) | 800*480 |  |
| DPI | 183 |  |
| Display color | Black, White, Red, Yellow, Blue, Green, Orange |  |
| NFC | 1. Complies with ISO/IEC 14443 Type A standard 2. Supports NFC NDEF data format with 200 bytes writable memory 3. Communication distance is reachable maximum 10 mm |  |
| LED Flash | Red, Blue, Green, Yellow, White, Cyan, Purple |  |
| Working life | 5-10 years (2-4 updates per day) |  |
| Battery Spec | 4*600mAh CR2450 |  |
| Changeable Battery | Battery Pack (replaceable) |  |
| Working Temperature (˚C) | 0~40 |  |
| Working Humidity (%RH) | 30~70 |  |
| Protection Level | IP54 |  |
| Certiﬁcation | ROHS,CE,FCC |  |
| RF Wireless Communicaiton Parameters | Working Frequency： 2402MHz ~ 2480MHz | System Throughput： Up to 6,000 labels per hour |

ESL Gateway

Model: AES-B001

The AES-B001 Electronic Shelf Label Control Base Station provides network access, tag communication, and update control for ESL systems. It integrates Wi-Fi, RJ45, Bluetooth, and multi-channel RF communication, supports desktop, vertical, ceiling-mounted, and other installation methods, and is suitable for retail stores, supermarket chains, warehouses, and centralized ESL management scenarios.

Features

Private communication protocol to ensure communication security and reduce interference;

High-speed bi-directional communication with ESL labels;

Supports Wi-Fi, RJ45 Ethernet, Bluetooth, and multi-channel RF communication;

Supports PoE, 12 V DC, and USB Type-C 5 V power input;

Compatible with PanPanTech ESL labels;

LED indicators display real-time operating status.

Hardware configuration

| AES-B001 specifications |  |
| --- | --- |
| Product Dimension(mm) | 155 x 155 x 35 (L x W x T) |
| Weight | approx. 332 g |
| Housing Color | White |
| Housing Material | ABS + PC |
| Mounting | Desktop, vertical, ceiling-mounted, and other installation methods |
| Main processor module | Quad-core ARM Cortex-A35 CPU, 1 GB DDR3 RAM, 16 GB eMMC storage |
| Display | 4-digit LED segment display |
| Indicator LEDs | Red: power; Yellow: operating status; Green: special status |
| Power supply module | 12 V DC 1 A / USB Type-C 5 V 2 A / PoE 48 V, max. 30 W |
| Network module | Wi-Fi 2.4 GHz AP/Station; RJ45 Ethernet; BLE 5.2 private protocol |
| RF module | Four-channel RF module, two transmit channels and two receive channels |
| Working Frequency | 2.4 GHz (2400 MHz - 2500 MHz), private protocol |
| Communication Distance | 0-25 m in non-open environments |
| Operating Current | <350 mA |
| Operating Temperature | 0 °C ~ 50 °C |
| Storage Temperature | -10 °C ~ 50 °C |
| Software Support | ESL management system / base-station control system; USB Type-C debugging |