PanPanTech

Classic Series Four-Color Electronic Shelf Label

Multi-Size Product Data Sheet

| Series | Classic Series |
| --- | --- |
| Product Category | Four-Color Electronic Shelf Label |
| Models Covered | AES-0213, AES-0266, AES-0290, AES-0420, AES-0750 |
| Version | V1.0 |
| Issue Date | 2026-05-28 |
| Document No. | PDS-AES-CLASSIC-V1.0 |

# 1. Series Overview

The PanPanTech Classic Series is a white-cover four-color electronic shelf label family designed for mainstream retail, supermarket, warehouse shelf, picking label, and product information display applications. It uses EINK e-paper display technology and retains the displayed image without continuous power.

The series supports 2.4 GHz / BLE 5.2 wireless updates via the ESL control base station. Recessed-display and mirror-cover structures may be supplied according to order configuration; this combined data sheet uses one specification set for both structures and does not distinguish them as separate models.

# 2. Model Range

| Model | Display Size | Resolution | DPI | Dimensions (mm) | Weight | Battery Capacity |
| --- | --- | --- | --- | --- | --- | --- |
| AES-0213 | 2.13" | 250 x 122 | 130 DPI | 74 x 35 x 12 mm | approx. 33 g | 1200mAh |
| AES-0266 | 2.66" | 296 x 152 | 125 DPI | 87 x 42 x 12 mm | approx. 39 g | 1200mAh |
| AES-0290 | 2.90" | 296 x 128 | 122 DPI | 92 x 43 x 12 mm | approx. 43 g | 1200mAh |
| AES-0420 | 4.20" | 400 x 300 | 120 DPI | 99 x 89.5 x 13 mm | approx. 83.5 g | 1800mAh |
| AES-0750 | 7.5" | 800 x 480 | 124 DPI | 176 x 123 x 9.5 mm | approx. 200 g | 3600mAh |

# 3. Common Display Specifications

| Item | Specification |
| --- | --- |
| Display Colors | Black / White / Red / Yellow |
| Display Technology | EINK e-paper display |
| Viewing Angle | >170° |
| Refresh Mode | Full refresh |
| Housing / Front Cover Color | White |
| Cover Structure | Recessed-display or mirror-cover configuration, not separated in this combined data sheet |

# 4. Communication & System

| Item | Specification |
| --- | --- |
| Wireless Protocol | BLE 5.2 private protocol via base station |
| Frequency Band | 2.4 GHz (2400-2500 MHz) |
| Transmission Range | 0-25 m in non-open environments |
| Update Method | Remote display updates via base station; OTA firmware upgrade supported |
| Management System | ESL management system / base station |
| Communication Security | Encrypted signaling with store-domain restriction |

# 5. Power & Electrical

| Item | Specification |
| --- | --- |
| Operating Voltage | 3.0 V DC (typical) |
| Static Sleep Current | <1.8µA |
| Scan Peak Current | <8mA |
| Communication Peak Current | <8mA |
| Display Refresh Current | <20mA |
| Battery Maintenance | Removable battery cover for battery replacement |

# 6. Individual Product Specifications

## 2.13-inch Classic Series ESL | Model: AES-0213

| Item | Specification |
| --- | --- |
| Product Dimensions (W x H x T) | 74 x 35 x 12 mm |
| Weight | approx. 33 g |
| Housing Material | ABS + PC |
| Case / Front Cover Color | White |
| Cover Structure | Recessed-display and mirror-cover configurations are available by order; this combined data sheet does not separate the two structures. |
| Display Size | 2.13 inch |
| Resolution | 250 x 122 px |
| Pixel Density | 130 DPI |
| Display Colors | Black / White / Red / Yellow |
| Display Technology | EINK e-paper display |
| Viewing Angle | >170° |
| Refresh Mode | Full refresh |
| Battery Type | 2 x CR2450 coin cells, 1200 mAh total |
| Battery Life | >5 years, based on 4 screen refreshes per day |
| Mounting | Standard mounting slot |
| Product ID | CODE128 barcode printed as the tag ID |
| Indicator LED | Tri-colour LED, red / green / blue |

## 2.66-inch Classic Series ESL | Model: AES-0266

| Item | Specification |
| --- | --- |
| Product Dimensions (W x H x T) | 87 x 42 x 12 mm |
| Weight | approx. 39 g |
| Housing Material | ABS + PC |
| Case / Front Cover Color | White |
| Cover Structure | Recessed-display and mirror-cover configurations are available by order; this combined data sheet does not separate the two structures. |
| Display Size | 2.66 inch |
| Resolution | 296 x 152 px |
| Pixel Density | 125 DPI |
| Display Colors | Black / White / Red / Yellow |
| Display Technology | EINK e-paper display |
| Viewing Angle | >170° |
| Refresh Mode | Full refresh |
| Battery Type | 2 x CR2450 coin cells, 1200 mAh total |
| Battery Life | >5 years, based on 4 screen refreshes per day |
| Mounting | Standard mounting slot |
| Product ID | CODE128 barcode printed as the tag ID |
| Indicator LED | Tri-colour LED, red / green / blue |

## 2.90-inch Classic Series ESL | Model: AES-0290

| Item | Specification |
| --- | --- |
| Product Dimensions (W x H x T) | 92 x 43 x 12 mm |
| Weight | approx. 43 g |
| Housing Material | ABS + PC |
| Case / Front Cover Color | White |
| Cover Structure | Recessed-display and mirror-cover configurations are available by order; this combined data sheet does not separate the two structures. |
| Display Size | 2.90 inch |
| Resolution | 296 x 128 px |
| Pixel Density | 122 DPI |
| Display Colors | Black / White / Red / Yellow |
| Display Technology | EINK e-paper display |
| Viewing Angle | >170° |
| Refresh Mode | Full refresh |
| Battery Type | 2 x CR2450 coin cells, 1200 mAh total |
| Battery Life | >5 years, based on 4 screen refreshes per day |
| Mounting | Standard mounting slot |
| Product ID | CODE128 barcode printed as the tag ID |
| Indicator LED | Tri-colour LED, red / green / blue |

## 4.20-inch Classic Series ESL | Model: AES-0420

| Item | Specification |
| --- | --- |
| Product Dimensions (W x H x T) | 99 x 89.5 x 13 mm |
| Weight | approx. 83.5 g |
| Housing Material | ABS + PC |
| Case / Front Cover Color | White |
| Cover Structure | Recessed-display and mirror-cover configurations are available by order; this combined data sheet does not separate the two structures. |
| Display Size | 4.20 inch |
| Resolution | 400 x 300 px |
| Pixel Density | 120 DPI |
| Display Colors | Black / White / Red / Yellow |
| Display Technology | EINK e-paper display |
| Viewing Angle | >170° |
| Refresh Mode | Full refresh |
| Battery Type | 2 x CR2450 coin cells, 1800 mAh total |
| Battery Life | >5 years, based on 4 screen refreshes per day |
| Mounting | Standard mounting slot |
| Product ID | CODE128 barcode printed as the tag ID |
| Indicator LED | Tri-colour LED, red / green / blue |

## 7.5-inch Classic Series ESL | Model: AES-0750

| Item | Specification |
| --- | --- |
| Product Dimensions (W x H x T) | 176 x 123 x 9.5 mm |
| Weight | approx. 200 g |
| Housing Material | ABS + PC |
| Case / Front Cover Color | White |
| Cover Structure | Recessed-display and mirror-cover configurations are available by order; this combined data sheet does not separate the two structures. |
| Display Size | 7.5 inch |
| Resolution | 800 x 480 px |
| Pixel Density | 124 DPI |
| Display Colors | Black / White / Red / Yellow |
| Display Technology | EINK e-paper display |
| Viewing Angle | >170° |
| Refresh Mode | Full refresh |
| Battery Type | 6 x CR2450 coin cells, 3600 mAh total |
| Battery Life | >5 years, based on 4 screen refreshes per day |
| Mounting | Standard mounting slot |
| Product ID | CODE128 barcode printed as the tag ID |
| Indicator LED | Tri-colour LED, red / green / blue |

# 7. Operating Environment

| Item | Specification |
| --- | --- |
| Operating Temperature | 0 °C to 50°C |
| Storage Temperature | -10 °C to 50°C |
| Operating Humidity | To be confirmed |

# 8. Software, Maintenance & Security

| Item | Specification |
| --- | --- |
| Main Controller | ARM Cortex-M0 @ 64 MHz, 512 KB Flash, 48 KB SRAM |
| Software Upgrade | OTA software update or upgrade via base station |
| Reliability | Continuous operation throughout lifecycle without restart |
| Software Functions | Key exchange, display update via base station, scheduled scanning, name-based scan filtering, battery voltage reporting, ambient temperature reporting, RSSI reporting, tag image update, LED flashing command, key removal, in-store base-station roaming, roll-call response, automatic calendar refresh, base-station time synchronization, display data storage, and fast page switching. |

# 9. Certifications, Warranty & Packaging

| Item | Specification |
| --- | --- |
| Certifications | To be confirmed |
| Warranty | To be confirmed |
| Unit Packaging | To be confirmed |
| Carton Quantity | To be confirmed |
| Carton Dimensions | To be confirmed |
| Net Weight | To be confirmed |
| Gross Weight | To be confirmed |

# 10. Compatible ESL Control Base Station

The following control base station is listed as a compatible system accessory for Classic Series ESL deployment.

| Item | Specification |
| --- | --- |
| Model | AES-B001 |
| Controller / Storage | Quad-core ARM Cortex-A35 CPU, 1 GB DDR3 RAM, 16 GB eMMC storage |
| Display | 4-digit LED segment display |
| Indicator LEDs | Three indicator LEDs: red for power, yellow for operating status, and green for special status |
| 12 V DC Input | DC barrel jack input, 12 V / 1 A |
| 5 V DC Input | USB Type-C input, 5 V / 2 A |
| PoE Input | Standard PoE Ethernet interface, 48 V, max. 30 W |
| Operating Current | <350mA |
| Wi-Fi | Built-in Wi-Fi module; supports AP and Station modes |
| RJ45 | Wired Ethernet connection via RJ45 |
| Bluetooth | Private protocol based on BLE 5.2; compatible with BLE 5.1 / BLE 5.0 / BLE 4.2 |
| RF Module | Integrated four-channel RF module: two transmit channels and two receive channels |
| Frequency Band | 2.4 GHz (2400 MHz - 2500 MHz), private protocol |
| Transmission Range | 0-25 m in non-open environments |
| Interfaces | Reset: Built-in reset button; USB Type-C: Used for DC 5 V power input or software debugging |
| Dimensions (L x W x T) | 155 x 155 x 35 mm |
| Weight | approx. 332 g |
| Housing Material | ABS + PC |
| Housing Color | White |
| Mounting | Desktop, vertical, ceiling-mounted, and other installation methods |

# 11. Remarks

- The specifications above are consolidated from the existing AES English product specification documents.

- Packaging, certification, humidity, warranty, and other fields not confirmed in the source documents remain marked as “To be confirmed”.

- Specifications are subject to update; the mutually confirmed contract, sample, or final technical document shall prevail.

- OEM / ODM customization is available, including housing color, printed logo, and management system UI.